| Document: Weather Data SOP | Author: Justin Kokoszka |
| --- | --- |
| Project: WXqaqc_Phase1 | Date: 2026-02-12 |
| Version: 1.0 |  |

## 1. Project Overview & Goals

The primary objective of the WXqaqc_Phase1 project is to transform raw weather data from 32 stations into a standardized dataset flagged for sensor specific physical and logical thresholds. The output of this project was designed to improve:

- Data Integrity: Validating sensor performance against physical limits and known dependencies.

- Operational Readiness: Providing field crews with accurate, up-to-date site histories to inform maintenance.

- Dissemination Readiness: Creating a clean, flagged dataset suitable for sharing with stakeholders.

Scope: This SOP covers the full lifecycle of Phase 1: from raw data compilation and metadata logging to QA/QC and final reporting. This project does not include statistical flagging, further statistical analyses, final data management, development of future procedures regarding field/qaqc, water temperature records, or specific data sharing.

## 2. Roles and Responsibilities

| Role | Responsibility |
| --- | --- |
| Principle Investigator (PI) | Provides high-level climatological guidance, defines the overall scientific scope, and approves final project procedures and deliverables. |
| Project Coordinator (PC) | Responsible for tracking project progress in the Project Plan Tracker (MS Loop); including roadblocks and assisting with the development and refinement of initial sensor thresholds. Also responsible for assisting in data technician role. |
| Data Manager (DM) | Responsible for supplying raw data files, executing the QA/QC scripts, managing the Sensor Thresholds Reference Table, and generating the final QA/QC Summary Reports. |
| Data Technician (DT) | Responsible for compiling raw data into the standardized naming convention, digitizing historical field notes, and populating the Station Metadata Logs. |

## 3. General Protocols

### 3.1 Project Tracking (MS Planner)

All project activities must be tracked in the Project Tracker.

- Granularity: Each Station is a unique "Task."

- Status Updates: The Task bucket must reflect the current stage (e.g., moved from Metadata Compilation to QAQC.

- Issues: If you run into a roadblock... move the task to the roadblock bucket and add detailed notes regarding the issue.

### 3.2 File Naming Convention

Strict adherence to the File Naming Protocol is mandatory to ensure consistency and standardization. Naming conventions can be found here.

NOTE: Any files that do not fit within the defined naming convention must be given to the PCs attention for a storage solution.

### 3.3 Data Management & Storage

All data must be stored in the designated directory structure within the NHG Group – Data Management shared drive. The project file structure can be found here.

- Project Location: MS TEAMS/NHG Group/Data Management/Shared/01_Projects/01_WXqaqc_Phase1

NOTE: Any files that do not fit within the defined file structure must be given to the PCs attention for a storage solution.

## 4. Project Workflow

This workflow follows a linear progression. Tasks are passed between the Data Manager (DM) and Data Technician (DT) using MS Loop as the tracker.

### Step 1: Data Compilation and Migration

Owner: Data Manager (DM)

Planner bucket: Not Started -> Metadata Log

- Source Collection:

  - The DM compiles all available historical files for a specific station (i.e. task).

- The “Data Dump”

  - The DM uploads ALL files (Raw data and Field Notes) into the data repository folder:

    - .../00_Data_Repository

      - NOTE: If you run into a roadblock... move the task to the roadblock bucket and add detailed notes regarding the issue.

      - NOTE: At this stage, filenames may be messy or inconsistent. This is expected.

- Task Hand-Off:

  - The DM locates the specific Station Task in MS Loop and moves the task from Not Started to the Metadata Logs bucket.

  - This signals to the Data Technician that the station is ready for processing.

### Step 2: File Organization and Renaming

Owner: Data Technician (DT)

Planner bucket: Metadata Log -> QAQC

- Task Retrieval:

  - The DT checks the Metadata Logs bucket for new stations.

- File Naming:

  - The DT navigates to the station 00_Data_repository. Every file must be renamed before it is moved, strictly following the File Naming Protocol.

    - NOTEs:

      - If you run into a roadblock... flag the task the ‘roadblock’ flag and write a short description of the issue. The PC will check and respond.

      - For stations with more than 15ish raw data files, renaming may not be practical, contact PC.

- File Migration:

  - Once renamed, the DT moves the files from the 00_Data_Repository into their respective sub-folders:

    - Raw data -> /02_Raw_Data

    - Field notes -> /01_Metadata/01_FieldNotesArchive

    - NOTEs:

      - If you run into a roadblock... flag the task the ‘roadblock’ flag and write a short description of the issue. The PC will check and respond.

      - If the date of download or visit is not available, use 99999999 as a default.

- Metadata Digitization:

  - The DT opens the Station Metadata Log (MetadataLog.xlxs) located in the /01_Metadata folder and populates the log based on the event type with the relevant details as specified below. See the equipment inventory list for equipment specifics.

    - GOAL: Not all metadata will be available; pre-2023 (i.e. pre fastfield) records may require unique solutions, contact PC with concerns.

    - NOTEs:

      - If you run into a roadblock... flag the task the ‘roadblock’ flag and write a short description of the issue. The PC will check and respond.

  - Event Types and required details:

    - Site Visit: Any visit to a site, including site initiation.

      - Date: Date of visit

        - YYYY-MM-DD

        - Unknown: Unknown

      - Time-in: Arrival time (PDT; 24hr)

        - HH:MM

        - Unknown: Unknown

      - Time-out: Departure time (PDT; 24hr)

        - HH:MM

        - Unknown: Unknown

      - Description: A brief description of the site visit.

      - Data/Visit/Script_ID: Unique identifier for site visit.

        - See the File Naming Protocol, for visit_ID naming conventions.

      - Flag

        - Specify a V for visit flag

      - FileName: File name for the reference field report.

      - FieldNotes:

        - Copy and paste all relevant field notes from the field report specific to the site visit.

    - Script Change: Any script changes, including the initial script.

      - Date: Date of script change.

        - YYYY-MM-DD

        - Unknown: Unknown

      - Description: A brief description of the script change.

      - Data/Visit/Script_ID: Unique identifier for script change.

        - See the File Naming Protocol, for Script_ID naming conventions.

      - Equip_model: Logger model

        - e.g., CR350

        - Unknown: -9999

      - Equip_manu: Logger manufacturer

        - e.g., Campbell Scientific

        - Unknown: -9999

      - Equip_serial: Logger serial number

        - e.g., 1379

        - Unknown: -9999

      - Equip_code: NHG equipment id

        - e.g., NHG0090

        - Unknown: -9999

      - Equip_asset: UNBC asset number

        - e.g., 9087

        - No Asset number: NULL

      - Flag: Flag to assist with qaqc

        - Assign an LR (logger restart) flag or a C (caution) flag with specific QAQC comments to note any significant consequences of the script change.

        - Default: LR

      - FileName: Name of script file

        - See the File Naming Protocol, for logger script naming conventions.

        - Unknown: -9999

      - FieldNotes: Copy and paste any notes from the field report specific to the data logger script.

    - Equipment Deploy: Any record of equipment being deployed at a station.

      - Date: Date of deployment.

        - YYYY-MM-DD

        - Unknown: Unknown

      - Description: A brief description of the equipment deployment.

      - Equip_model: model

        - e.g., CR350

        - Unknown: -9999

      - Equip_manu: manufacturer

        - e.g., Campbell Scientific

        - Unknown: -9999

      - Equip_serial: serial number

        - e.g., 1379

        - Unknown: -9999

      - Equip_code: NHG equipment id

        - e.g., NHG0090

        - Unknown: -9999

      - Equip_asset: UNBC asset number

        - e.g., 9087

        - No Asset number: NULL

      - Height_depth_cm: sensor height/depth in cm.

        - Unknown: -9999

      - File_Name: file name of the reference field report.

        - Unknown: -9999

      - Field_Notes:

        - Copy and paste any notes from the field report specific to the equipment deployment.

    - Equipment Decom: Any record of equipment being decommissioned from a station.

      - Date: Date of equipment decommission.

        - YYYY-MM-DD

        - Unknown: Unknown

      - Equip_model: model

        - e.g., CR350

        - Unknown: -9999

      - Equip_manu: manufacturer

        - e.g., Campbell Scientific

        - Unknown: -9999

      - Equip_serial: serial number

        - e.g., 1379

        - Unknown: -9999

      - Equip_code: NHG equipment id

        - e.g., NHG0090

        - Unknown: -9999

      - Equip_asset: UNBC asset number

        - e.g., 9087

        - No Asset number: NULL

      - File_Name: file name of the reference field report.

        - Unknown: -9999

      - Field_Notes:

        - Copy and paste any notes from the field report specific to the equipment decommission.

    - Data download:

      - Date: Date of data download.

        - YYYY-MM-DD

        - Unknown: Unknown

      - Description: A brief description of the data download. Can be left as none.

      - Data/Visit/Script_ID: Unique identifier for raw data.

        - See the File Naming Protocol, for Data_ID naming conventions.

      - Equip_model: Logger model

        - e.g., CR350

        - Unknown: -9999

      - Equip_manu: Logger manufacturer

        - e.g., Campbell Scientific

        - Unknown: -9999

      - Equip_serial: Logger serial number

        - e.g., 1379

        - Unknown: -9999

      - Equip_code: NHG equipment id

        - e.g., NHG0090

        - Unknown: -9999

      - Equip_asset: UNBC asset number

        - e.g., 9087

        - No Asset number: NULL

      - Flag: Flag to assist with qaqc

        - Assign a C (caution) flag with specific QAQC comments to note any significant consequences of the script change.

      - FileName: Name of script file

        - See the File Naming Protocol, for raw data naming conventions.

        - Unknown: -9999

      - FieldNotes: Copy and paste any notes from the field report specific to the data download.

  - DT checks the station task and completes the metadata log checklist, then moves the task to the qaqc bucket.

### Step 3: QAQC

Owner: Data Manager (DM)

Planner bucket: QAQC -> Completed

This stage converts raw data into a "Tidy" format with quality flags. The process is strictly hierarchical: Metadata -> Physical -> Logical -> Dependency.

NOTE: Some stations will have long datasets, in such cases contact the PC.

NOTE: If you run into a roadblock... move the task to the ‘roadblock’ bucket and write a short description of the issue. The PC will check and respond.

- The DM monitors the QAQC bucket in MS Planner for available stations.

- Inputs:

  - Navigate to the station folder.

  - Locate raw files in /02_Data_Raw.

  - Open the MetadataLog.xlsx from /01_Metadata.

- Compilation and Metadata:

  - Using the Metadata Log, identify the unique data_id for each raw file.

- Level 1: Standardization and Concatenation:

  - Standardize each raw data file:

    - Append the data_id as a column to every row to maintain traceability back to the source file and equipment list.

    - Add the station code

    - Timestamp Formatting: Format: YYYY-MM-DD HH:MM.

    - Check the timezone and convert to PDT if required, add the final UTC offset (-7)

    - Force the time index to strict 15-minute intervals.

    - Rename columns to match the project standard and generate a corresponding _Flag column for every data parameter (e.g., Temp_Air_C_Flag). Initialize all as empty.

  - Concatenate all raw data files:

    - Append all standardized raw data.

    - Insert "empty" rows for missing 15-minute steps. Flag these rows as M (Missing).

- Level 2: Flagging

Flags must be applied in this exact order to ensure dependencies work correctly.

Layer 1: Metadata Flags

- Input: Metadata Log (Visit History & Comments).

- Action:

  - Flag data during Site Visits as V.

    - Specify protocol

  - Apply C (Caution) flags based on specific technician comments (e.g., smp instead of avg).

  - Flag LR where logger restarts occur

Layer 2: Physical Flags (Hard and Soft Limits)

- Input: Ref_Sensor_Thresholds.xlsx and MetadataLog.xlxs

- Action: Compare data against the threshold limits defined for each specific sensor model over the time-period it was deployed. Use the default if no sensor details are available over any period.

  - R (Range) =  Outside sensor operating range

  - C (Caution) = Outside sensor cautionary limit

  - E (Error) = Sensor Error, specifics depend on sensor model (Notes in Ref_Sensor_Thresholds.xlxs)

- NOTE: If the sensor is not on threshold list contact the PC.

Layer 3: Logical Flags

- Input: Ref_Sensor_Thresholds.xlsx (Logical Flags).

- Action: Run logic checks such as:

  - Z (Zero) = > 0 at night

  - NV (No value) = no wind direction if no wind speed, no strike distance if no strikes

  - SF (Snow Free) = estimated snow free period June to Sept

Layer 4: Dependency Flags

- Action: Check the "Dependencies" column in the Reference Table.

- Note: This step must happen last so it can use the physical and logical flags as reference

  - PT (Panel Temp): Panel temperature flagged: R

  - BV (Battery Voltage): Battery voltage flagged: R

  - DF (Dependency Fail): One or more dependent variable is flagged: R, E, DF, M

    - Dependent variable is calculated

  - DC (Dependency Caution): Refer to Notes column

  - T (Tilt): Refer to Notes column

  - DZ (Divide by Zero): Refer to Notes column

- Finalization

- Save Output: Save the fully processed file to /03_Data_Tidy. See the File Naming Protocol, for tidy data naming conventions.

- Task Completion: Move the Station Task in MS Loop to the Done bucket.