QAQC intensive Workshop – Conference Centre 6-305

Objective: To design and validate a QAQC procedure for NHG weather station data.

Day 1: Thursday (Jan 22) – Foundations, Metadata, and Data Ingestion

Theme: Standardization and preparing the “Raw” data.

09:00 – 09:30 | Review Schedule and Set-up

- Itinerary: Review workshop itinerary

- Access: Confirm data access

09:30 – 10:00 | Session 1: Workshop kick-off and Alignment

- Meeting: Join Stephen in meeting teams meeting online

- Overarching Goals:

  - Why are we collecting this data? What will it be used for? (e.g., climate monitoring, trends, etc.)

  - Does our current protocol follow FAIR data principles? (Finable, Accessible, Interoperable, and Reusable)

- Immediate Goals: Confirm the specific “definition of done” for the workshop.

  - Review:

    - Proposed QAQC phases

      - Phase 1: Metadata, Format, and Consistency Check

      - Phase 2: Parameter QAQC

    - Proposed Flag Tests and Criteria

    - Proposed Parameter Thresholds

- Scope Check: Ensure everyone agrees on the specific stations/parameters being tested during the technical sessions.

- Future Goals: Discuss future directions:

  - Scalability: Can this QAQC process handle more stations and more parameters?

  - Accessibility: Who needs to access the data in the future? Do we need an API or a public dashboard? Do we need an internal dashboard?

  - Archiving: Long-term storage strategies and version control for the data.

- Impact on QAQC:

  - How do these future goals influence the development of our QAQC protocol today? (e.g., how strict do our thresholds need to be? What metadata is useful for users and the field crew?)

- Deliverables:

  - QAQC Project Plan

  - WX Data Management Plan

10:00 - 11:00 | Session 2: Inventory and Standardization

- Station Review: Review of the full list of current and historical WX stations.

- Parameter Audit: List of all collected parameters (Primary and Secondary).

- Prioritization: Assign priority levels to stations (Critical and Secondary).

- Standardization: Define the rigid list of standardized parameter names to be used across all code.

- Deliverables:

  - Station list

  - Parameter list with standardized names

11:00 – 12:00 | Session 3: Metadata Compilation and Linkage

- Metadata Audit: Compile records for test site (Historical field notes, Sensor IDs/History, Logger Codes, Pictures, Historical data)

- Linkage Logic: Plan the logic to link sensors to loggers and logger codes to specific datasets.

- Gap Analysis: Identify where metadata is missing or disconnected from the data files.

- Deliverables:

  - Metadata summary

12:00 – 13:00 | Lunch Break

13:00 – 15:00 | Technical Session A: Ingestion and Phase 1 QAQC Protocol

- Rapid Prototyping (Coding):

  - Script the file loading process.

  - Header and Timestamp Check: Validate headers, validate date/time format, assign time zone, fill missing date/time, and validate consistency.

  - Assign Essential Metadata: Assign metadata records (station_id, data_id, visit_id, logger_id, code_id, sensor_id, units, measure_type (sample, total, average)

    - NOTE: Need to identify a proper data structure for this.

- Deliverables:

  - Functional phase 1 prototype script

  - Phase 1 tidy files for test site

15:00 – 16:30 | Component Review: Ingestion and Field Consistency

- Evaluation: Review the failures found during the timestamp/header checks.

- Field Implications: Feedback for the field crew regarding logger setup (e.g., clock syncing protocols, header programming, field notes) to minimize errors.

- Data Storage: Review current data storage protocols and future improvements (e.g., metadata, field notes, photos, raw data, tidy data; Are all these data Finable, Accessible, Interoperable, and Reusable?)

- Deliverables:

  - List of phase 1 failures for test site

  - Feedback report for field crew

16:30 - 17:00 | Day 1 Wrap-up

- Review of progress and deliverables

| Task | Complete | Assigned to | Due |
| --- | --- | --- | --- |
| QAQC Project Plan |  |  |  |
| Data Management Plan |  |  |  |
| Station List |  |  |  |
| Parameter List with Standard Names |  |  |  |
| Metadata Summary |  |  |  |
| Phase 1 prototype script |  |  |  |
| Phase 1 tidy file for test site |  |  |  |
| Phase 1 summary |  |  |  |
| Phase 1 feedback for field crew |  |  |  |

Day 2: Friday Jan 23rd – Analysis and Reporting

Theme: Thresholds and Outputs

09:00 - 09:30 | Review Goals and Set-up

- Goals: Review workshop goals for today.

- Access: Confirm data and file access.

9:30 – 12:00 | Technical Session B: Historical Analysis and Thresholds

- Flag Criteria: Review QAQC tests and flag criteria

  - Historical check: function used to calculate the max, min, max Δ, min Δ occurring in each record; used for spike/jump tests and extreme value tests

    - Min and max Δ are 99.99 and 0.01 percentiles; ensures large jumps are investigated by the user

| Flag | Description | Symbol | Parameter |
| --- | --- | --- | --- |
| Pass | Good | P | All |
| No value | Data point missing | M | All |
| Spike | Compare x to x-1 and x+1. If change between x and x+1 AND x and x-1 is greater than 99.5% of change from historical data, then flag | S | Temp, RH, Snow, Pressure |
| No change | Constant value over time | NC | Temp (4-steps), RH (10-steps), wind (10-steps), wind direction, Pressure |
| Jump | Compare x to x+1. If absolute difference is greater than 99.5% found in historical data, then flag | J | Temp, RH, Snow, Pressure |
| Extremes | Data that falls outside historical min and max | E | Temp, RH, Rain, wind, snow, pressure |
| Should be NULL | Wind direction should be NULL when wind speed is 0 | NV | Wind direction |
| Snow free* | Snow free months | SF | Snow |
| Night | Value should be non-zero between dusk and dawn | Z | Radiation |
| Error* | Indicates an instrument error | ER | ALL |
| Threshold exceeded | Data falls outside max or min thresholds | T | ALL |

* User confirmation should be implemented and may need investigation of field notes

List of potential dependencies:

- Tilt – if tilt is flagged then associated data should be flagged?

- Ice – wind speed/direction no change + below 0 temp; precip when temp below 0 and observed precip for length of steps forward?

- Data Set: Compile phase 1 data for test site

- Manual Check: For each parameter,

  - Script and run initial thresholds (see above and below)

  - View flagged data using interactive plot and table

    - If data looks good, change flag to P

    - If data looks like a possible error, leave the flag

      - When in doubt, leave the flag

    - Check snow parameters during the summer to better estimate snow times

Proposed Thresholds

| Parameter | Threshold | Parameter | Threshold |
| --- | --- | --- | --- |
| Max Δ temp in 15min | ±5°C | Max Δ wind speed in 15min | No limit |
| Min temp | +50°C | Max avg wind speed | 50m/s |
| Max temp | -50°C | Max Δ snow depth in 15 min | ±100mm |
| Max Δ RH in 15min | ±20% | Max snow depth | 5m* |
| Max rainfall in 15min | 20mm | Max Δ pressure in 15min | 10hPa |
| Max pressure | 1050hPa | Min Pressure | 850hPa* |
| First snowfall | Oct 31* | Last snowfall | June 01* |
| Max battery voltage | ? | Min battery voltage | ? |
| Max Δ SW↓ in 15min | ? | Max Δ SW↑ in 15min | ? |
| Max SW↓ | ? | Max SW↑ | ? |
| Min SW↓ | ? | Min SW↑ | ? |
| Max Δ LW↓ in 15min | ? | Max Δ LW↑ in 15min | ? |
| Max LW↓ | ? | Max LW↑ | ? |
| Min LW↓ | ? | Min LW↑ | ? |

*Check for site specific details; NOTE: We could have dependent variables for flags (e.g., tilt)

- Initial QAQC Save:

  - Once all parameters have been checked manually, export the Initial historical qaqc file

- Validation:

  - Does the code correctly flag anomalies? Are thresholds too low or too high?

- Deliverables:

  - Phase 2 qaqc data for test site

  - Phase 2 summary for test-site

    - Threshold ranges, qaqc notes, and validation comments

  - Phase 2 Field crew feedback: Could this be used as resource for field work initial data checks?

12:00 – 13:00 | Lunch Break: NHG Weekly Lunch

13:00-14:30 | Technical Session B Continued

- Complete deliverable

14:30-16:00 |Technical Session D: Reporting and Implementation Plan

- Output Design:

  - QAQC Report: Design template for the immediate session report (flags, stats, gaps)

  - Cumulative Report: Design the summary report structure (long-term data)

- Field Operations: Finalize the “Field Data Collection” updates. What MUST change in the field to support this QAQC process?

- Deliverables:

  - Phase 2 QAQC report template

  - Cumulative QAQC report template

  - Finalize field crew suggestion document

16:00 – 17:00 | Final Review and Roadmap

- Workflow review: Full flow review

  - Data collection -> Storage -> QAQC -> Reporting -> Dissemination

- Next Steps: Assign tasks for project implementation/documentation and field crew briefing

- Deliverables:

  - Project Plan

| Task | Complete | Assigned to | Due |
| --- | --- | --- | --- |
| Phase 2 data for test site |  |  |  |
| Phase 2 summary for test site |  |  |  |
| Phase 2 qaqc template |  |  |  |
| Cumulative qaqc summary template |  |  |  |
| Field crew suggestions document |  |  |  |